dopey2 description, spin

This document is not ready.

This document covers methods used for visualizing and doing initial anlysis of data from Prodigy.

Data is loaded with the dopey.load() method.

Measurement types

The methods are compatible with spin EDC, MDC, and Map. Spin EDCs are reccorded with analyzer scan mode FixedAnalyzerTransmission, and MDCs and Maps are are reccorded with analyzer scan mode FixedEnergy. An EDC is reccorded with fixed deflector angles, a MDC is reccorded by scanning one deflector, and a Map is reccorded by scanning both deflectors. Arbitrary line scans, when stepping the deflectors together, are possible but not yet covered by the dopey.load() method.

Examples

EDC

data = dopey.load(spin_edc.xy)

Data type:

spin_edc

Data, axes, and parameters:

intensity           counts/s                 shape=(8, 66)

axis0               Kinetic energy (eV)      shape=(66,)

parameter0          NegativePolarity         shape=(8,)

The .intensity is an array with, in this case, eight EDCs where each curve has 66 data points. .axis0 is the energy axis and .parameter0 contains the magnetisation polarity of the target for each curve.

Note: due to the unfortunate naming convention, NegativePolarity = -1 means what we refere to as positive polarity, which is pointing along y in the lab system for coil 1 and along z for coil 2.

MDC

data = dopey.load(spin_mdc.xy)

Data type:

spin_mdc

Data, axes, and parameters:

intensity           counts/s                 shape=(8, 43)

axis0               ShiftY [a.u.]            shape=(43,)

parameter0          NegativePolarity         shape=(8,)

parameter1          ShiftY [a.u.]            shape=(43,)

The .intensity is an array with, in this case, eight MDCs with each curve has 43 data points. .axis0 is the deflector axis (in this case we deflect along y) and .parameter0 contains the magnetisation polarity of the target for each curve. .parameter1 is the same as .axis0.

Map

data = dopey.load(spin_map.xy)

Data type:

spin_map

Data, axes, and parameters:

intensity           counts/s                 shape=(8, 22, 20)

axis0               ShiftX [a.u.]            shape=(22,)

axis1               ShiftY [a.u.]            shape=(20,)

parameter0          NegativePolarity         shape=(8,)

The .intensity is an array with, in this case, eight maps where each map is a 2220 array (x×y). .axis0  and .axis1 are the deflector axes and .parameter0 contains the magnetisation polarity of the target for each map.

Methods

The following methods can be used for quick analysis of the spin data:

asymmetry(D = data, **kwargs)

polarization(**kwargs)

asymmetry()

Calculates the asymmetry, etc. and returns a new data object. For e.g. a spin EDC measurement the data object contains:

data = dopey.load(spin_edc.xy)

asym = dopey.asymmetry(data)

asym.arrays

Arrays:

intensity           shape=(8, 66)

axis0               shape=(66,)

intensity_off       shape=(66,)

intensity_on        shape=(66,)

asymmetry           shape=(66,)

component_plus      shape=(66,)

component_minus     shape=(66,)

The .intensity_off and .intenity_on arrays contain the mean intensities for the NegativePolarity = -1 and NegativePolarity = 1 respectivly, which are used to calculate the asymmetry in .asymmetry. The two arrays .component_plus and .component_minus is the distributed total intensity (generally refered to as component intensities).

Example:

To manipulate and analyze the data, first have a look at the curves:

_ = dopey.plot(data)

The first curve (c = 0) is an outlier and we ignore it by passing the attribute exclude (a list containing curves to exclude, in this case only 0) to the asymmetry() method. We also chose to normalize the intensities at the low kinetic energy side by passing normp and normpn.  Passing normp = 0 and normpn = 5 means that each curve is normalized by the mean intensity of the 5 first points.

asym = asymmetry(data, exclude = [0], normp = 0, normpn = 5)

The result is plotted by passing various keyword arguments as True to the plot() method.

_ = dopey.plot(asym, asymmetry = True)

_ = dopey.plot(asym, component = True)

Manipulating and ananlyzing MDC and Map is done in the same or similar way.

polarization()

This methods takes data from the asymmetry() method and calculates the spin polarization. For a complete calculation of the 3D spin vector we need four measurements:

- rotator +1, 	coil 1

- rotator -1, 	coil 1

- rotator +1, 	coil 2

- rotator -1, 	coil 2

With those four measurements the spin vector is completely determined. A complete calculation of the 2D in-plane spin vector requires two measurements:

- rotator +1, 	coil 2

- rotator -1, 	coil 2

However, due to the target being slightly angled (15°) it might not be possible to determine the out-of-plane polarization without having first determined the in-plane polarization.

If we are sure that there is no in-plane polarization we can correctly determine the out-of-plane polarization through one measurement only, using coil 1 and either of rotator +1 or -1. If the in-plane component is expected to be small one can get a good estimate of the out-of-plane component from two measuements with coil 1, one with rotator +1 and one with rotator -1.

We need to do four measurements if the polarization is unknown or the in-plane component is larger than not negligable.

Px, Py and Pz are calculated by:

Px=-AR+C2-AR-C22S

Py=AR+C2+AR-C22S

Pz=1cosθAR+C1+AR-C121S-sinθ2Px

where AR+C1, AR-C1, AR+C2, and AR-C2 are the asymmetries determined from the combinations of coil and rotator selections, S is the Sherman value, and θ is the target angle (15°).

Example, determining the in-plane spin:

c2rp = dopey.load(spin_edc_1.xy)

c2rm = dopey.load(spin_edc_2.xy)

a_c2rp = dopey.asymmetry(c2rp)

a_c2rm = dopey.asymmetry(c2rm)

p = dopey.polarization(c2rp = a_c2rp, c2rm = a_c2rm)

dopey.plot(p)